# General Description

# UHF RFID is a high performance UHF RFID integrated reader. It is designed upon fully self-intellectual property. Based on proprietory efficient DSP algorithm, it supports fast tag read/write operation with high identification rate. It can be widely applied in many RFID application systems such as logistics, access control, anti-counterfeit and industrial production process control system.

# FEATURES

- Self-intellectual property;

- Support ISO18000-6B, ISO18000-6C(EPC C1G2) protocol tag;

- 902~928MHz frequency band(frequency customization optional);

- FHSS or Fix Frequency transmission;

- RF output power up to 30dbm(adjustable);

- 12dbi antenna optional with effect distance up to 10m*;

- Support auto-running, interactive and trigger-activating work mode;

- Low power dissipation with single +9 /+12 DC power supply;

- Support RS232, RS485, Wiegand interface;

* Effective distance depends on antenna, tag and environment.

CHARACTERISTICS

- Absolute Maximum Rating

| ITEM | SYMBOL | VALUE | UNIT |
| --- | --- | --- | --- |
| Power Supply | VCC | 16 | V |
| Operating Temp. | TOPR | -10~+55 | ℃ |
| Storage Temp. | TSTR | -20~+75 | ℃ |

- Electrical and Mechanical Specification

Under TA＝25℃，VCC＝+9V unless specified

| ITEM | SYMBOL | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| Power Supply | VCC | 8 | 9 | 12 | V |
| Current Dissipation | IC |  | 350 | 650 | mA |
| Frequency | FREQ | 902 |  | 928 | MHz |

# Interface

| Item | Comment |
| --- | --- |
| Red | +9V |
| Black | GND |
| Yellow | Wiegand DATA0 |
| Blue | Wiegand DATA1 |
| Purple | RS485 R+ |
| Orange | RS485 R- |
| Brown | GND |
| White | RS232 RXD |
| Green | RS232 TXD |
| Grey | Trigger input（TTL level） |

* Optional model named as UHFReader18TCP with TCP/IP interface is also available.